Vacant Unit Maintenance Operational Guidelines

July 2026

OFFICIAL

# More information

To find out about housing options visit the Housing website or contact your local Housing Office.

To receive this publication in an accessible format contact Homes Victoria enquiries@homes.vic.gov.au.

This guideline contains some links to internal resources which will not be accessible for external parties reading this document.

Authorised and published by the Victorian Government, 1 Treasury Place, Melbourne.

© State of Victoria, Department of Families, Fairness and Housing, July 2026.

ISBN: 978-1-76171-204-3 (pdf/online/MS word)

Available on the on Services Providers website.

# Revision history

Chapter 2: Vacant Unit Maintenance v1.2 – Retired December 2024

| Version | Amended section | Effective | Details |
| --- | --- | --- | --- |
| 1.0 |  | November 2017 | Incorporation of version control table Accessible format |
| 1.2 | Mandatory works for vacant properties | April 2018 | Amended to reflect that gas appliances are serviced as part of mandatory works |

Vacant unit guide v2.0 – Retired December 2024

| Version | Amended section | Effective | Details |
| --- | --- | --- | --- |
| 1.0 | First publication | 29 March 2021 | Date of issue. |
| 2.0 | Revised | 7 July 2023 | Moved to DFFH template. The option for Head contractors to scope vacant works has been removed. Further information provided about raising Residential Tenancies Act safety checks |

Vacant Unit Maintenance Operational Guidelines

| Version | Amended section | Effective | Details |
| --- | --- | --- | --- |
| 1.0 | First publication | December 2024 | Combined Chapter 2: Vacant unit maintenance v1.2 and Vacant unit guide v2.0 into a single comprehensive guideline. Updates from previous versions: Removal of guidance that standard vacant unit works include safety and compliance checks. New sections have been introduced to provide updated and comprehensive guidance on vacant unit maintenance. |
| 1.2 | 4.1 Reletting restrictions | June 2025 | Expanded guidance on the management and application of reletting restrictions, including detailed oversight and approval procedures. Integrated standalone section 6.5 Upgrade Referral Requests into the reordered Reletting Restrictions section, which has been moved from section 4.1 to section 5. |
| 1.3 | - Throughout | March 2026 | Sections 3.1, 3.3 and 3.4 have been consolidated into Section 3, which has been renamed from Occupational health and safety to Safety and inspection requirements. Content previously in Section 5.2 Asset Management oversight and approval has been renamed Vacancy durations and relocated to Section 10 Referral guide and assessment: Asset Management Branch (now Section 10.4). The table Reletting restrictions requiring Asset Management oversight and approval, originally part of Section 5.2, has been removed and incorporated into the Referral guide and assessment: Asset Management Branch content. A new Section 10.3 Reletting restriction and vacancy management has been created from the remaining relevant Section 5.2 content, retaining information on reletting restrictions requiring Asset Management Branch review before implementation, and clarifying local office responsibilities during the vacancy period. Section 5.3 Raising a reletting restriction for review and approval has been renamed Pre‑referral requirements and relocated under Section 10 Referral guide and assessment: Asset Management Branch (now Section 10.1). Section 5.4 CSHS Manager oversight and approval of referrals to Asset Management has been renamed CSHS Manager oversight and approval and relocated under Section 10 Referral guide and assessment: Asset Management Branch (now Section 10.2). Section 6 Goods left behind has been removed, as it solely redirected staff to the Goods Left Behind Operational Guidelines for instruction. Removing it streamlines the document and avoids unnecessary cross‑referencing. Section 9 Vacant unit referrals to Asset Management Branch have been introduced. Section 10 Referral guide and assessment: Asset Management Branch has been introduced. Tables have been introduced throughout the document to improve clarity and usability. Section 9 Move‑in Tenancy Condition Reports has been relocated to become a subsection under Vacant unit handback inspection (now Section 7.1). As a result of the amendments to Section 5 and subsequent sections, the document’s section numbering has been updated, as detailed above. |
| 1.4 | 5.1 Closing reletting restrictions 5.2 HiiP tenancy task list 5.3 General manager oversight 6 Vacant unit work 8 Guide to vacant unit checks 9 Vacant unit referrals to Asset Management Branch 9.1 Reletting restrictions applied by Asset Management Branch 10.4 Vacancy durations | July 2026 | Updated: Section 4.1 Contractor extension requests to improve readability. Sections 5.1 and 5.2 to reflect automatic closure of reletting restrictions in HiiP, including staff responsibilities. Section 5.3 to clarify manager oversight of reletting restrictions. Section 6 and 8 to provide information and guides regarding minimum rental standards. Sections 9 and 9.1 to clarify the Asset Management Branch’s roles and responsibilities. Section 10.4 to link vacancy durations to reletting restrictions and specify which restrictions close automatically and those requiring manual intervention. |

# Contents

More information	2

1	Revision history	3

2	Contents	6

3	Overview	7

4	Safety and inspection requirements	7

4.1	Properties presenting a risk of contamination	8

5	Target timeframes for vacant unit maintenance	9

5.1	Contractor extension requests	9

6	Reletting restrictions	10

6.1	Reletting restriction management in HiiP	10

6.2	General manager oversight of reletting restrictions	11

7	Vacant unit work	11

7.1	Minimum rental standards	11

7.2	Mandatory lock changes	12

7.3	Standard work	13

7.4	Additional work	14

8	Vacant unit handback inspections	14

8.1	Tenancy Condition Reports	15

9	Guide to vacant unit checks	16

10	Vacant unit referrals to Homes Victoria	19

10.1	Reletting restrictions applied by Homes Victoria	20

11	Referral guide and assessment - Asset Management Branch	22

Asset intent	22

Condition based referral check	23

Asset intent descriptions	26

11.1	Pre referral requirements	28

11.2	CSHS Manager oversight and approval	29

11.3	Reletting restrictions and vacancy management	30

11.4	Vacancy durations	30

# Overview

Vacant unit maintenance refers to the routine tasks and checks performed to ensure a rental property is clean, safe, functional, and ready for a new renter after the previous renter has moved out. The objectives of vacant unit maintenance are to:

keep the property in a suitable and compliant condition while it is unoccupied

prepare the unit to meet the functional and safety needs of the next renter

maintain the unit so it meets all safety, health, and minimum rental standards.

# Safety and inspection requirements

When conducting vacant unit inspections, staff must take appropriate precautions to protect their safety and minimise exposure to potential hazards. The following table outlines safety requirements that apply when inspecting vacant units.

Table 1: Safety requirements for vacant unit inspections

| Requirement | Description |
| --- | --- |
| Condition of entry due to unsafe conditions | Do not enter or proceed with the vacant unit inspection if: the property poses any immediate threat to health or safety illegal occupants are identified unsafe conditions are observed such as: unstable flooring severe structural damage hazardous materials like asbestos cluttered or obstructed pathways. there are signs of contamination: methamphetamine pipes other drug paraphernalia, or evidence of pests and so on. Refer to Section 3.1 Properties presenting a risk of contamination for more information. |
| Pre‑entry risk check | Assess the exterior and surroundings for hazards before entering (for example, animals, damage, unsecured items, signs of illegal activity). |
| Announce entry | Knock and clearly announce entry to alert anyone inside and assess immediate risks before fully entering (for example, animals, persons present, hazards). |
| Time spent inside | Remain in the property only as long as necessary to complete the inspection to minimise exposure to potential hazards. |
| Personal protective equipment | Wear appropriate Personal Protective Equipment, including: sturdy footwear shoe covers gloves masks, and eye protection, where necessary. |
| Avoid placing personal items | Do not place personal items, such as folders, handbags, car keys, on furniture, fixtures, or surfaces to prevent contamination from dust, allergens, or other potential hazards in the property. |
| Refrain from touching surfaces without gloves | Do not touch surfaces or objects without gloves to minimise exposure to contaminants that may be present. |
| Vacant unit checks guide | Refer to Section 8 Guide to vacant unit checks to assist in supporting compliance with minimal rental standards during inspections. |
| Printed room identifiers | Use printed room identifiers when taking photos to clearly correlate each image with the corresponding part of the property. |
| Document findings thoroughly | Record all observations and findings in detail to maintain a comprehensive inspection report. |

## Properties presenting a risk of contamination

If a vacant property is assessed as potentially contaminated, based on prior information or observations during the vacant unit inspection, the external locks must be changed by the head contractor. The key must then be securely stored (for example, in a lockbox at the property or the local office).

No further vacant works may proceed until the local office has completed all required work, including any necessary testing, specialist assessment, or remediation.

Table 2: Common contamination risks and required actions

| Type | Detail |
| --- | --- |
| Asbestos | If asbestos‑containing materials are suspected or identified, staff must follow all safety protocols and refer the property to specialist contractors for assessment and remediation. |
| Pest infestations | When signs of pests (for example, rodents or insects) are identified, staff must arrange professional pest control services. |
| Mould | Where mould is detected, its extent should be assessed, and specialist remediation engaged to remove it safely. |
| Needles or drug paraphernalia | Staff must not touch or handle syringes or drug‑related items. These must be reported immediately and removed by biohazard services. |
| Rubbish and debris | Excessive rubbish or waste must be cleared by professional cleaning or waste removal services. |

For more information on managing methamphetamine contamination in vacant public housing properties, refer to the interim guidelines available on the Public Housing Resources SharePoint, Vacants page (internal link).

# Target timeframes for vacant unit maintenance

Vacant unit maintenance is a key performance indicator (KPI), tracking the efficiency of the process. Offers to new renters are typically made during this period, supporting readiness for occupation once maintenance is completed.

The target timeframe for raising vacant unit maintenance is within 2 working days of either the end of a Residential Rental Agreement, in accordance with the Residential Tenancies Act 1997 (RTA), or a Victorian Civil and Administrative Tribunal decision determining the property has been abandoned. Once the vacant works are scoped and raised in HiiP Repairs, the head contractor has 14 calendar days to complete the works.

## Contractor extension requests

Full assessment of contractor extension requests is mandatory to prevent operational delays and maintain contractor accountability. Delays to vacant unit turnaround times cause significant operational impacts, including:

extended vacancy periods, increasing costs and slowing allocations

pressure on service delivery and renters waiting for housing

ongoing contractor performance issues if delays are not addressed

### Considerations for approving extension requests

When assessing a contractor’s extension request, staff must apply the requirements outlined in the following table:

Table 3: Requirements for assessing extension requests

| Requirements | Description |
| --- | --- |
| Verify supporting evidence | Confirm the contractor has provided enough information to explain the delay (for example, photographs, reports, specialist assessments). |
| Consult relevant staff | Seek input from staff such as Field Services Officers to help validate the request and understand site conditions. |
| Record the decision | Document the reasons for approving or declining the request in HiiP Repairs to support transparency, consistency, and auditability. |
| Manager approval | All extension requests must be approved by Team Managers or equivalent delegated officers. |

Where delays are not justified, staff must enforce liquidated damages in line with contractual requirements and operational impacts. Refer to Section 4.1.2 Applying liquidated damages for more information.

### Applying liquidated damages

Liquidated damages help hold contractors accountable for delays and recover costs arising from missed deadlines. They may be applied when:

the contractor fails to meet contractual completion dates without valid justification

delays cause financial loss to the department

application of damages aligns with the terms and conditions of the contract.

When a delay is unjustified, once the vacant unit works are completed and accepted following a handback inspection, staff must:

Calculate the penalty period: Determine the exact number of calendar days between the contractual completion date and the actual completion date.

Send a contractor memorandum: Issue a contractor memo via HiiP Repairs stating the total number of unapproved delay days, and the intent to deduct liquidated damages from the final payment.

# Reletting restrictions

A reletting restriction is applied to a vacant property in HiiP to prevent it from being offered to new renters before it is ready to be re-let. When applied, the property is flagged as unavailable for reletting (vacant untenantable). This restriction is applied when outstanding issues must be addressed, such as:

managing goods left behind

removing illegal occupants

undertaking extensive maintenance works, or

finalising strategic property decisions.

When a reletting restriction is necessary, staff must select from the defined list of reletting restriction types and use them strictly as intended. Applying defined reletting restriction types supports accurate reporting, provides clear visibility into extended vacancies, and helps prioritise management actions to reduce turnaround times.

While staff apply and manage most reletting restrictions, Homes Victoria’s Asset Management Branch, Relocations, and Leasing functions also apply them to support capital planning, property strategy, and compliance works.

For a complete list of reletting restriction types and associated delegates, refer to the Reletting Restriction Matrix available on the Public Housing Resources SharePoint, Vacants page (internal link).

For more information on how reletting restrictions are applied and managed, refer to:

Section 5.1 Reletting restriction management in HiiP

Section 5.2 General manager oversight of reletting restrictions

Section 9.1 Reletting restrictions applied by Homes Victoria

Section 10.3 Reletting restrictions and vacancy management

Section 10.4 Vacancy durations

## Reletting restriction management in HiiP

HiiP’s tenancy task list supports reletting restriction management by automatically sending email notifications to the controlling office mailbox when a reletting restriction is:

closed manually by staff

automatically closed by HiiP when its end date is reached

automatically closed when a contract reaches handover.

withdrawn (cancelled)

rejected by Asset Management

approaching expiry (5 days prior).

The notification includes the property details and restriction type. Staff must use these email alerts alongside standard reletting restriction reports to:

identify the asset

complete follow-up tasks, and

update system records.

Mailbox administrators must promptly forward these alerts to authorised delegates to prevent action delays.

For more information on HiiP reletting restriction management, refer to the relevant HiiP functionality documents on the Public Housing Resources SharePoint HiiP resources page (internal link).

## General manager oversight of reletting restrictions

Team Managers, Client Support and Housing Services (CSHS) Managers, or their equivalents, are responsible for applying, closing, and regularly monitoring reletting restrictions. Their oversight prevents properties from being returned for re-letting before they are ready. Depending on local vacancy volumes, managers must review restricted properties on a daily to weekly basis. Oversight activities include:

tracking approval requests, reletting restriction end dates and any related follow-ups.

reviewing the reletting restriction types to confirm they are current, correct, and used in line with the Reletting Restriction Matrix.

closing or removing restrictions when properties are ready to be re-let, including those closed automatically by HiiP upon expiration.

monitoring automatic email notifications.

updating end dates where necessary to prevent premature automatic closures before required actions are finished.

To identify vacant properties with restrictions nearing expiration, refer to the Expiring Reletting Restrictions - Vacant Properties report on the Housing Reporting Toolbox (internal link).

# Vacant unit work

## Minimum rental standards

All vacant units must meet the 15 minimum rental standards set out in Schedule 4 of the Residential Tenancies Regulations 2021 before they can be re-let.

These standards are based on the actual physical condition of the property. Staff help check compliance by completing a structured walkthrough of the property where they collect and record evidence of its condition. This helps determine what work is required so that it is RTA compliant when the property is re-let.

To confirm the property meets the standards, staff must collect 3 types of evidence for each checklist item:

Table 4: Minimum rental standards - Evidence requirements

| Evidence type | Required action | Non-technical verification standard |
| --- | --- | --- |
| Visual evidence | Date stamped photography | Capture wide-angle, date-stamped digital photographs of each room to document overall condition of the property. Capture close-up photographs of any defects, along with a wide-angle photograph showing the defect’s location in the room. |
| Documentary evidence | Compliance logging | Raise a responsive maintenance work order in HiiP Repairs immediately for any failed item. Use clear and plain language to describe the issue. Record the status of all 15 minimum rental standards in the Tenancy Condition Report. This creates a record at the start of the tenancy. |
| Functional evidence | Physical checks | In addition, with the Vacant Unit Check Guide, complete these checks: Check all external door locks by turning the key to make sure they lock from the outside and open from the inside without a key. Flush all toilets to make sure they empty fully, refill properly and do not leak or block, Turn on all taps in the kitchen, bathrooms, and laundry to check water pressure, flow and hot water. Turn on the fixed heater in the main living area to make sure it works. Turn on the cooktop and oven to check they heat up correctly. Switch all light on and off to make sure they work in every room, hallway and corridors. Open and close all windows to make sure they move easily and lock properly. Open and close all curtains and blinds to check they move smoothly, cover the window and that cords are secured safely. Open and close all internal doors, cupboards and drawers to make sure they work properly. |

## Mandatory lock changes

Changing the locks is a mandatory requirement for every vacant unit to maintain the property's security and safety. This practice is not only a best practice but also a regulatory requirement aimed at protecting renter safety. The following table outlines the SOR codes associated with the mandatory lock change work:

Table 5: Mandatory changing of locks SOR codes

| Location | SOR code | Description |
| --- | --- | --- |
| Front Door | GH055 | Re-pin 1 x cylinder & supply 2 keys alike (for all external doors) |
| Other Doors | GH056 | Re-pin cylinders to other doors |

## Standard work

Standard vacant unit works generally include tasks such as:

cleaning

adjusting and tightening all screens on doors, cupboards, hinges, and fittings and

cleaning spouting and downpipes.

However, before staff raise these works, they should assess whether these types of standard works are necessary. For instance, while cleaning is generally regarded as standard vacant unit work, a visual inspection may reveal that not every room requires cleaning.

Common SOR cleaning codes staff may consider raising as part of their standard vacant unit works include:

Table 6: Common SOR codes and descriptions

| Location | SOR code | Description |
| --- | --- | --- |
| Multiple locations | CR070 | Clean wipe & vacuum all surfaces |
| Multiple locations | CR155 | Clean carpet by vacuuming- Rate per unit |
| Multiple locations | GA205 | Check adjust and tighten all screws to all doors, cupboard doors, handles and fittings and so on |
| Multiple locations | CR202 | Remove contact from walls and cupboards only if required |
| Multiple locations | GA205 | Check and tighten all screws to all cupboards |
| Multiple locations | CR158 | Steam clean carpets - per room (more than 1 room) |
| Multiple locations | CR157 | Steam clean carpet - 1 room only |
| Kitchen | CR175 | Clean stove |
| Kitchen | CR188 | Clean kitchen complete - does not include floor |
| Kitchen | CR250 | Clean and scrub floors |
| Kitchen | CR195 | Clean exhaust fan only - not to be used w/cleaning |
| Laundry | CR210 | Clean linen laundry landing cupboards and built in robe |
| Laundry | CR211 | Clean laundry fixtures complete -including exhaust fan |
| Laundry | CR212 | Wash laundry complete- ceiling/walls and so on |
| Bathroom | CR220 | Clean bathroom complete- include floor |
| Toilet | CR230 | Wash and scrub toilet complete including exhaust fan and floor |
| Hall | CR262 | Wash hall |
| Lounge | CR252 | Wash lounge and family room |
| Bedrooms | CR256 | Wash bedroom |
| External | RS270 | Clean out spouting |
| External | RS260 | Clean down pipe |

## Additional work

When considering additional maintenance works during a vacant property inspection, staff should apply a value‑for‑money approach. Table 7 outlines the key principles to guide decision‑making.

Table 7: Principles for assessing additional maintenance needs

| Principle | Description |
| --- | --- |
| Repair over replacement | Prioritise repairing items whenever feasible, rather than opting for full replacement. |
| Minimal aesthetic work | Limit work to the minimum necessary for aesthetic improvement, focusing on essential repairs that enhance the property’s appearance without unnecessary cost. |
| Targeted floor covering replacement | If carpet or vinyl in a room is damaged or marked beyond acceptable levels, replace only the affected area rather than the entire room. |
| Selective wall repair | Patch and repaint only the specific wall that is damaged or unable to be cleaned; avoid repainting the full room unless necessary. |
| Standardised door handle installation | If internal door handles do not match the departmental standard and one is missing, install a standard-compliant handle on the specific door only. |
| Odour management in carpets | If strong odours persist after steam cleaning, identify which rooms or sections of carpet are affected and replace only those areas. |

# Vacant unit handback inspections

As the property nears completion, the contractor will schedule a date and time for a vacant unit inspection. Staff are responsible for conducting a thorough inspection of the vacant unit alongside the contractor, reviewing all raised repair items. This includes cross-checking that all works have been completed to a satisfactory standard before accepting the unit as ready to be re-let.

Expectations for staff during hand back inspections include:

Thorough review: Inspect all areas of the unit to confirm repairs align with what was agreed upon in the scope of works. Pay attention to common issues like:

cleaning,

safety features

compliance with minimum rental standards

quality of work.

This includes checking for functionality (for example,, working plumbing, locks, and appliances).

Staff must not accept a hand-back of the property if works remain incomplete or do not meet required standards. Clearly document any outstanding or unsatisfactory work and communicate this to the contractor for rectification.

## Tenancy Condition Reports

Move-in Tenancy Condition Reports are a mandatory requirement in the sign-up process, as stipulated by the RTA. These reports must be completed by staff during the vacant unit handback or prior to re-letting a property. They support accurate documentation of the property's condition and help establish its state at the time of occupancy.

For more information, refer to the Tenancy Condition Reports Operational Guidelines.

# Guide to vacant unit checks

| Property address | Click or tap here to enter text. |
| --- | --- |

Table 8: Vacant unit checks

| Item category | Description | Inspected | Work required | Not applicable |
| --- | --- | --- | --- | --- |
| Bathroom facilities (Minimum rental standard) | Check that the bathroom/s has a washbasin and the shower or bath are connected to a steady supply of hot and cold water. | ☐ | ☐ | ☐ |
| Locks (Minimum rental standard) | Check that all external entry doors have working deadlocks that lock with a key from the outside and open easily from the inside without a key. | ☐ | ☐ | ☐ |
| Toilets (Minimum rental standard) | Check that the toilet flushes properly, connects to a working waste system and is located in a suitable, separate room. | ☐ | ☐ | ☐ |
| Kitchen facilities (Minimum rental standard) | Check that the kitchen has a food preparation area, a sink with running water, a stovetop with at least 2 working burners and a working oven if one is provided. | ☐ | ☐ | ☐ |
| Laundry facilities (Minimum rental standard) | If a laundry is provided, check that there is a working wash trough or washing machine taps with hot and cold water. | ☐ | ☐ | ☐ |
| Structural soundness (Minimum rental standard) | Check walls and floors for noticeable slopes or cracks and gaps wider than 10 millimetres wide. | ☐ | ☐ | ☐ |
| Mould and damp (Minimum rental standard) | Check for visible mould or damp caused by building or water issues, including stains or damage on ceilings, walls, floors, coverings, and cupboards. | ☐ | ☐ | ☐ |
| Electrical and switchboard safety (Minimum rental standard) | Check that there is no exposed wiring anywhere on the property and that the main switchboard is modern with flip-switch styled circuit breakers and residual current device (RVD) safety switches, rather than old ceramic pull-out fuses. | ☐ | ☐ | ☐ |
| Ventilation (Minimum rental standard) | Check that habitable rooms, bathrooms, toilets, and laundries have ventilation through an openable window, an external door, or a working exhaust fan. | ☐ | ☐ | ☐ |
| Windows (Minimum rental standard) | Check that all external windows designed to open have working latches, locks, or security fittings to keep the property secure. | ☐ | ☐ | ☐ |
| Lighting (Minimum rental standard) | Check that all rooms, corridors, and hallways have working artificial lights for night-time and access to natural light during the day. | ☐ | ☐ | ☐ |
| Heating (Minimum rental standard) | Check that there is a fixed heater in the main living area, that it turns on, runs without unusual noises or smells and shows no visible damage to the unit or control. | ☐ | ☐ | ☐ |
| Window coverings (Minimum rental standard) | Check that all bedroom and living room windows have curtains or blinds. They must block light and provide privacy from the outside. | ☐ | ☐ | ☐ |
| Window covering anchors (Minimum rental standard) | Check that any corded blinds or curtains have a safety anchor. The anchor must be fixed at least 1.6 metres above the floor to keep children safe. | ☐ | ☐ | ☐ |
| Rubbish and recycling bins (Minimum rental standard) | Check that the property has matching rubbish and recycling bins. They must be council-approved, in good working condition, and used only by the property. | ☐ | ☐ | ☐ |
| Security screen doors | Check that security screens are fitted to all external doors where required and that they open and close properly. | ☐ | ☐ | ☐ |
| Hot water system | Check for leaks, pooling, or dripping around the water heater, and check that hot water is available at all taps at a steady temperature. | ☐ | ☐ | ☐ |
| Smoke alarms | Check that smoke alarms are installed and test them using the test button to confirm they work. | ☐ | ☐ | ☐ |
| Floor coverings | Check that internal floor coverings are secure, and free from loose edges or physical trip hazards. | ☐ | ☐ | ☐ |
| Tap functionality | Check that all taps turn on and off without dripping and provide steady water pressure. | ☐ | ☐ | ☐ |
| Pest infestation | Check for signs of pests or damage, such as mud tunnels on walls, hollow or brittle timber, small dark droppings in cupboards or accumulated animal waste. | ☐ | ☐ | ☐ |
| Appliances | Check that all supplied appliances turn on, run without unusual noises or smells and show no visible exterior damage. | ☐ | ☐ | ☐ |
| Carbon monoxide alarms | If carbon monoxide alarms are required, check that they are installed and test them using the test button to confirm they work. | ☐ | ☐ | ☐ |
| Telephone line and NBN connection | Where required, check that a telephone line and National Broadband Network (NBN) connection are present. | ☐ | ☐ | ☐ |
| Non-standard modifications safety | Check for structures and fixtures such as homemade shelving, temporary room dividers, non-standard door locks, or backyard structures that look unstable or block walkways. | ☐ | ☐ | ☐ |
| Kitchen – cupboards and drawer’s functionality | Check that all kitchen cupboards and drawers open and close easily and have handles fitted. | ☐ | ☐ | ☐ |
| Internal doors with hardware | Check that all rooms have doors with handles and that internal doors do not have unapproved locks or latches fitted. | ☐ | ☐ | ☐ |
| Flywire screens | Check that at least 1 window in each room has a flywire screen fitted and that screens are in good repair. | ☐ | ☐ | ☐ |
| Bathroom – cupboards and drawer’s functionality | Check that bathroom cupboards and drawers open and close easily and have handles fitted. | ☐ | ☐ | ☐ |
| Letterbox, clothesline and shed | Check that the property has a working letterbox, clothesline, and garden shed or storage locker, if applicable. | ☐ | ☐ | ☐ |
| Tree proximity to external walls | Check that no trees are within 1 metre of external brick walls and that trees or shrubs do not block access or press on structures. | ☐ | ☐ | ☐ |
| Swimming pools | Check that there are no swimming pools or external spas present on the property. | ☐ | ☐ | ☐ |

# Vacant unit referrals to Homes Victoria

When assessing a vacant property for escalation, staff must identify which works fall within local office responsibility and capacity, and which require involvement from Homes Victoria’s Asset Management Branch.

The following tables outline the different types of works, clarify responsibility for each, and provide examples to assist in determining whether a referral is required. Staff must also refer to Section 10 Referral guide and assessment – Asset Management Branch when determining responsibility and prior to submitting a referral.

Table 9: Capital works (Asset Management Branch responsibility)

| Category | Description |
| --- | --- |
| Definition | Significant repairs, upgrades or replacements that restore or improve the long-term function of the property. These works usually exceed the financial and operational capacity of local offices. |
| Delivery pathways | Programmed capital works delivered annually by Homes Victoria. Minor capital works delivered locally under the Schedule of Rates (SOR) or as Not on Schedule (NOS) items. |
| Examples of capital works | Structural repairs: major roof replacement, structural wall repairs, extensive flooring replacement. Compliance works: fire safety upgrades, asbestos removal, accessibility modifications. Extensive damage repairs: fire‑damaged properties, major vandalism repairs, methamphetamine/mould/pest contamination remediation. Terrain and site works: retaining walls, major drainage works, significant landscaping or earthworks. |
| Referral requirement | Vacant properties requiring this level of work must be referred to the Asset Management Branch for assessment. |

Table 10: Non-capital works (Local office responsibility)

| Category | Description |
| --- | --- |
| Definition | Day-to-day maintenance and minor repairs needed to keep a property safe, functional, and ready to be re-let. These works do not extend the asset’s life and are carried out locally. |
| Delivery pathway | Local housing offices raise work orders under the SOR for vacant maintenance and fair wear-and-tear repairs. |
| Examples of non‑capital works | General maintenance: lock changes, repairing windows, fixing taps. Cleaning and waste removal: rubbish removal, pest control, internal and external cleaning. Minor repairs: patching walls, securing loose fixtures, partial gutter replacements, minor flooring repairs. Safety and security: fence repairs, reattaching cupboard doors, replacing smoke alarm batteries, fixing broken tiles or benchtops. |
| Referral requirement | These works should be completed locally. They do not require referral to the Asset Management Branch. |

In some situations, routine maintenance may uncover underlying issues that require Capital Works. For example, a leaking roof may be repaired locally, but a Field Services Officer inspection may reveal that the roof is significantly deteriorated and requires full replacement. When this occurs:

local offices should complete the immediate non‑capital maintenance needed to address the presenting issue (for example, repairing the active leak).

if further investigation identifies major or extensive works, the local office must document the findings and submit a referral to the Asset Management Branch for assessment.

When a property is referred to the Asset Management Branch, the branch is responsible for:

managing reletting restrictions in HiiP and tracking properties through vacancy stages to support full compliance with the Reletting Restrictions Matrix. Refer to Section 5 Reletting restrictions for more information.

reviewing fixed-term reletting restrictions prior to their scheduled end date.

updating, extending, or closing fixed-term reletting restrictions within required timeframes to prevent unintended expiration.

collaborating with operational business areas to support timely implementation.

## Reletting restrictions applied by Homes Victoria

Homes Victoria applies reletting restrictions and monitors vacant properties for an expected vacancy duration that is determined by the referral reason. Table 11 below outlines the vacancy types, referral reasons and vacancy durations that Homes Victoria is responsible for:

Table 11: Properties referred to Homes Victoria - by referral reason

| Referral reason | Reletting restrictions | Vacancy duration |
| --- | --- | --- |
| Disposal Major Works Internal Product Code Change | Under review - disposal strategy low rise assets Under review - disposal strategy specialist accommodation assets Under review - disposal strategy high rise assets Under review - major works Under review - product type change Handover incomplete - further works required | 30 days |
| Internal Product Type Change | Stock transfer to RGS program | 60 days |
| Relocation Lease Management | For relocation projects Under review - lease management Rental general stock for general lease | 60 days |
| Major Works Lease management | Handover delayed Lease handback | 90 days |
| Internal Product Type Change Major Works Disposal | Stock transfer to non RGS programs Asset reviewed - pending upgrade Awaiting conversion General upgrade Major mobility modification requests Building movement/structural Hazardous/contamination remediation works Disposal strategy - development pipeline Disposal strategy - sell pipeline | 185 days |
| Major Works | Fire replacement | 365 days |
| Disposal | Disposal strategy - approved future masterplan Disposal strategy - approved development Disposal awaiting demolition Disposal awaiting sale | No end date |

Reletting restrictions with a fixed vacancy duration will automatically close in HiiP at the end of the relevant vacancy duration. The property will return to the local office, indicating that the property is available to let.

Homes Victoria must review the property’s vacancy type before changing any reletting restriction. If the property is not ready to be re-let, Homes Victoria must close the current reletting restriction and immediately create a new reletting restriction with the correct business rules, rather than extending the existing one. This is because the vacancy duration is fixed and does not update when a restriction is extended.

Before any property is returned to the local office, Homes Victoria must contact the local office to confirm they are ready to allocate the property.

Reletting restrictions with no end date will not automatically close and must be managed manually.

For more information on reletting restriction types, referral reasons and vacancy durations, refer to the Reletting Restriction Matrix available on the Public Housing Resources SharePoint, Vacant page (internal link).

# Referral guide and assessment - Asset Management Branch

| Property address | Click or tap here to enter text. |
| --- | --- |

| Property PIN | Land PIN | Year built | Cladding type | Demand and supply |
| --- | --- | --- | --- | --- |
| Click or tap here to enter text. | Click or tap here to enter text. | Click or tap here to enter text. | Click or tap here to enter text. | Choose an item. |

## Asset intent

Select the relevant asset intent. Refer Asset intent descriptions for more information.

| Asset intent | Referral requirements |
| --- | --- |
| Choose an item. | Where the asset intent is ‘Hold and Improve’ or ‘Hold and Maintain’, the referral must be submitted via the OurService Upgrade Referral Request eform (internal link). Where the asset intent is ‘Review’, ‘Sell’ or ‘Redevelop’, the referral must be submitted via the OurService Disposal Referral Request eform (internal link). |

## Condition based referral check

Is the property affected by any of the following conditions? (Select all that apply). If any listed condition applies, use the OurService Upgrade Referral Request eform (internal link).

| Condition category | Yes / No | Referral required | Criteria / referral requirements | Supporting documents attached | Referral notes |
| --- | --- | --- | --- | --- | --- |
| Fire damage | Yes ☐ No ☐ N/A ☐ | Yes | All fire‑damaged properties must be referred via OurService. | Yes ☐ No ☐ N/A ☐ | Click or tap here to enter text. |
| Renter damage | Yes ☐ No ☐ N/A ☐ | Case‑by‑case | Renter damage may be identified where deterioration or damage is caused by the actions, misuse, or neglect of renters or their visitors. Referral applies when: the damage impacts structural integrity, safety, or RTA compliance specialist trades or significant rectification are needed. | Yes ☐ No ☐ N/A ☐ | Click or tap here to enter text. |
| Vandalism / squatters | Yes ☐ No ☐ N/A ☐ | Case‑by‑case | Vandalism or squatter damage may be identified where unauthorised individuals have caused damage to the property, affecting security or habitability. Referral applies when: the damage is significant or affects essential services, safety, or structural elements specialist rectification or security upgrades are required. | Yes ☐ No ☐ N/A ☐ | Click or tap here to enter text. |
| Methamphetamine contamination | Yes ☐ No ☐ N/A ☐ | Case‑by‑case | Methamphetamine contamination is identified when local testing confirms the presence of contamination requiring remediation. Referral applies when: remediation involves significant strip‑out works contamination impacts habitability or compliance. | Yes ☐ No ☐ N/A ☐ | Click or tap here to enter text. |
| Early degradation | Yes ☐ No ☐ N/A ☐ | Case‑by‑case | Early degradation may be identified where the level of deterioration is beyond what the local office can reasonably repair. Referral applies when: specialist trades or structural rectification are needed the issue is likely to escalate without broader asset intervention. In addition, the deterioration must not be: due to renter damage caused by fire, contamination, or vandalism the result of an isolated incident, but instead systemic or building‑related. | Yes ☐ No ☐ N/A ☐ | Click or tap here to enter text. |
| Structural repairs | Yes ☐ No ☐ N/A ☐ | Case‑by‑case | Structural repair needs may be identified when damage or deterioration affects the building’s structural integrity or safety. Referral applies when: works relate to foundations, load‑bearing elements, framing, or other structural components engineering assessment or specialist trades are required the extent of structural work exceeds the high‑cost vacant threshold. Structural issues must not be managed locally where safety or structural integrity is compromised. | Yes ☐ No ☐ N/A ☐ | Click or tap here to enter text. |
| Full upgrade is required due to major issues not related to fire, renter damage or contamination | Yes ☐ No ☐ N/A ☐ | Case‑by‑case | A full upgrade may be identified when the property has significant deterioration or functional failure requiring comprehensive renewal. Referral applies when: multiple building elements have reached end‑of‑life or require replacement the scale of works exceeds local office capacity specialist or coordinated upgrade planning is required. | Yes ☐ No ☐ N/A ☐ | Click or tap here to enter text. |
| Age related upgrade | Yes ☐ No ☐ N/A ☐ | No (unless unsafe) | Age‑related upgrade need may be identified where the property is safe and habitable but approaching the point where major renewal will soon be required. Referral is not required unless safety is compromised. Where the property remains compliant with the RTA, staff should: re-let the property submit a request for inclusion in future upgrade programs via OurService. | Yes ☐ No ☐ N/A ☐ | Click or tap here to enter text. |

## Asset intent descriptions

| Asset intent | Description |
| --- | --- |
| Hold and improve | This intent applies where the property meets Homes Victoria’s asset objectives and continues to be a productive asset to be maintained and potentially improved. |
| Hold and maintain | This intent applies where the property is older and close to the end of its asset lifecycle but requires minimal maintenance or periodical renewal. At the end of its lifecycle, the site may be suitable for redevelopment or sale. The property should be maintained to meet the requirements of the Residential Tenancies Act, including Minium Rental Standards. |
| Redevelop | This intent applies where the land should be retained as it meets Homes Victoria’s overall objectives. Redevelopment may: retain and redevelop the site for social housing, including planning, demolition and construction of new dwellings; or retain the land for commercial objectives, such as commercial retail or ground lease options. |
| Sell | This intent applies where the property does not adequately meet Homes Victoria’s asset objectives or is not financially prudent to invest in (for example, lower‑demand locations, limited or expensive redevelopment options, neighbourhood fatigue, site limitations, or restrictive/prohibitive planning controls) |
| Review | This intent applies where a more detailed review is required to determine the appropriate intent for the property. Examples include: the property is at end of life but still occupied development opportunities exist on contiguous sites that are occupied alignment is required with precinct development or neighbourhood strategy the property is part of broader work to confirm intent for a precinct or walk‑up block and depends on a realisation plan significant work is needed to determine intent for a complex precinct. |

Properties should not be referred to the Asset Management Branch when:

the issue can be addressed through standard make‑safe or minor repairs

costs fall within local office repair capacity

the CSHS manager has not reviewed and approved the referral

relevant information required has not been provided (for example, year built, cladding type)

pre-referral vacant unit work is not complete

the required documentation for the condition category has not been attached to the referral.

For subsequent procedures refer to:

Section 10.1 Pre referral requirements

Section 10.2 CSHS Manager oversight and approval

Section 10.3 Reletting restrictions and vacancy management

## Pre referral requirements

When properties require substantial interventions beyond the scope of routine maintenance or local office capacity, staff and managers should determine if the Asset Management Branch’s involvement is required to support appropriate escalation.

Before raising and escalating a reletting restriction and an OurService referral, including product type changes, staff must first initiate the necessary vacant unit works, which include:

Table 11: Required actions before raising a reletting restriction and referral

| Requirement | Description |
| --- | --- |
| Secure the property | Change all locks and place keys in the on-site key safe. |
| Remove all rubbish and goods | Remove all rubbish and personal goods from internal and external areas, including sheds |
| Arrange a basic clean | Raise work orders for a clean based on the property’s condition and complete the clean as required. |
| Address suspected meth contamination | If methamphetamine contamination is suspected, obtain an Occupational Hygienist report and remediation quotation. |
| Prepare cost estimates | Provide an estimate for vacant unit works, referencing the Schedule of Rates where applicable. |

Staff must also compile comprehensive information for a thorough and well-documented referral, including:

Table 12: Required documentation for escalation

| Documentation | Description |
| --- | --- |
| Explanation of reletting restriction | Detailed notes outlining the reasons for the restriction, including assessments and decisions made. |
| Field Service Officer report | Assessment of the property’s condition. |
| Council Emergency Orders | Include where relevant, particularly for fire-damaged properties. |
| Methamphetamine reports and costs (if applicable) | Certified Occupational Hygienist report, including the testing report and remediation action plan Field Services Officer property report, including the Property Survey Report (PCA001) Methamphetamine remediation cost estimate worksheet. Remediation cost estimates must not include the cost of vacated maintenance works or the removal of rubbish that can be disposed of through normal municipal waste services. |
| Photographs | Internal and external images clearly showing the issues prompting escalation. |
| Other relevant documentation | Any additional reports or evidence relating to the property’s condition. |
| Vacancy checklist | Completed checklist confirming the property’s status and condition. |
| Referral guide and assessment - Asset Management Branch | Completed referral guide to the Asset Management Branch, where applicable. |

Once the above has been completed, staff must simultaneously:

Table 13: Actions to complete after prerequisites are finalised

| Action | Description |
| --- | --- |
| Apply and escalate the reletting restriction in HiiP | Apply and escalate the relevant reletting restriction within 24-48 hours of identifying the need for Asset Management involvement. Include the OurService reference number in the comments to support accurate tracking and follow-up. |
| Initiate an OurService referral | Submit a referral through the OurService Catalogue under Housing Services, selecting the request type appropriate to the situation. |
| Submit for CSHS Manager review and approval | Forward the reletting restriction and referral through the HiiP and OurService workflows to the CSHS Manager (or equivalent) for review and approval. |

## CSHS Manager oversight and approval

Before approving a referral, CSHS Managers must first determine if works can be completed locally. This includes confirming that:

Table 14: CSHS Manager requirements before approving a referral

| Requirement | Description |
| --- | --- |
| Assess local capability | Confirm whether the work can be completed locally, including: whether repairs are minor, and within local cost limits |
| Review supporting evidence | Review: Field Service Officer reports cost estimates, and risk assessments to confirm they clearly show whether the work can be managed locally or requires escalation. |
| Confirm reletting restriction status | Validate a RAM-endorsed reletting restriction has been applied in HiiP, with accurate and complete notes outlining the reason for escalation. |
| Validate referral pathway | Validate the referral aligns with the correct path based on: asset intent property type, and cladding considerations. |
| Confirm OurService referral completeness | Make sure all relevant information has been entered into the OurService form, with an accurate description and the correct request type selected. |
| Verify required documentation is attached | Confirm submission includes: Field Service Officer report(s) council Emergency Order (if applicable) methamphetamine remediation reports and costings (if required) internal and external photographs showing the issues any other relevant reports or documentation completed vacancy checklist |
| Record recommendation and decision | Provide recommended upgrade reason(s), include a brief description of the property’s condition, and approve or decline the referral in a timely manner. |

If these conditions are met, the referral should not be approved and the work managed locally.

## Reletting restrictions and vacancy management

If a referral is accepted by Homes Victoria, the associated reletting restriction may be applied. The local office retains responsibility for vacancy management tasks, including:

securing the property in the event of any incidents

maintaining the grounds, including grass cutting

taking appropriate action against illegal occupants, if necessary.

## Vacancy durations

When a property is referred to Homes Victoria for assessment or works, it may remain vacant for an extended period depending on the referral reason and the type of works or strategy involved. Vacancy durations for referred properties are set out in the Reletting Restriction Matrix, which should be used as the primary reference for vacancy durations, reletting restriction types and referral reasons.

Reletting restrictions with a set vacancy duration will automatically close in HiiP when their end date is reached. Reletting restrictions with no fixed end date will not close automatically and must be closed manually by authorised delegates once the property is ready to be re-let.

For vacancy duration details for specific referral reasons and reletting restriction types, refer to the Reletting Restriction Matrix available on the Public Housing Resources SharePoint, Vacants page (internal link).

http://www.housing.vic.gov.au

http://www.housing.vic.gov.au/contact-a-housing-office

https://providers.dffh.vic.gov.au

https://dhhsvicgovau.sharepoint.com/sites/Publichousingresources/SitePages/vacants.aspx?web=1

https://dhhsvicgovau.sharepoint.com/sites/Publichousingresources/SitePages/vacants.aspx

https://dhhsvicgovau.sharepoint.com/sites/Publichousingresources/SitePages/HiiP-User-Guides.aspx

https://tableau.reporting.dhhs.vic.gov.au/t/HousingPerformance/views/HPLandingPage/LandingPage?:embed=y&:showShareOptions=true&:display_count=no&:showVizHome=no#1

https://providers.dffh.vic.gov.au/tenancy-management-operational-guidelines

https://dhhsvicgovau.sharepoint.com/sites/Publichousingresources/SitePages/vacants.aspx

https://ourservice.support.vic.gov.au/ourservice?id=sc_cat_item&sys_id=5bfb744adb6a2110ddf5bc56f39619b1&sysparm_category=dfeb09fe1bc52110af5d98ef0d4bcb79.

https://ourservice.support.vic.gov.au/ourservice?id=sc_cat_item&sys_id=edc96f261b2e6510af5d98ef0d4bcbf9

https://ourservice.support.vic.gov.au/ourservice?id=sc_cat_item&sys_id=5bfb744adb6a2110ddf5bc56f39619b1&sysparm_category=dfeb09fe1bc52110af5d98ef0d4bcb79.

https://dhhsvicgovau.sharepoint.com/sites/Publichousingresources/SitePages/vacants.aspx