Vacant Unit Maintenance Operational Guidelines

March 2026

OFFICIAL

Vacant Unit Maintenance Operational Guidelines March 2026

## More Information To find out about housing options visit the Housing website http://www.housing.vic.gov.au or contact your local Housing Office http://www.housing.vic.gov.au/contact-a-housing-office. To receive this publication in an accessible format contact Homes Victoria enquiries@homes.vic.gov.au. This guideline contains some links to internal resources which will not be accessible for external parties reading this document. Authorised and published by the Victorian Government, 1 Treasury Place, Melbourne. © State of Victoria, Department of Families, Fairness and Housing, March 2026. ISBN: 978-1-76171-030-8 (pdf/online/MS word) Available on the on Services Providers website https://providers.dffh.vic.gov.au.

# Revision history

Chapter 2: Vacant Unit Maintenance v1.2 – Retired December 2024

| Version | Amended section | Effective | Details |
| --- | --- | --- | --- |
| 1.0 |  | November 2017 | Incorporation of version control table Accessible format |
| 1.2 | Mandatory works for vacant properties | April 2018 | Amended to reflect that gas appliances are serviced as part of mandatory works |

Vacant unit guide v2.0 – Retired December 2024

| Version | Amended section | Effective | Details |
| --- | --- | --- | --- |
| 1.0 | First publication | 29 March 2021 | Date of issue. |
| 2.0 | Revised | 7 July 2023 | Moved to DFFH template. The option for Head contractors to scope vacant works has been removed. Further information provided about raising Residential Tenancies Act safety checks |

Vacant Unit Maintenance Operational Guidelines

| Version | Amended section | Effective | Details |
| --- | --- | --- | --- |
| 1.0 | First publication | December 2024 | Combined Chapter 2: Vacant unit maintenance v1.2 and Vacant unit guide v2.0 into a single comprehensive guideline. Updates from previous versions: Removal of guidance that standard vacant unit works include safety and compliance checks. New sections have been introduced to provide updated and comprehensive guidance on vacant unit maintenance. |
| 1.2 | 4.1 Reletting restrictions | June 2025 | Expanded guidance on the management and application of reletting restrictions, including detailed oversight and approval procedures. Integrated standalone section 6.5 Upgrade Referral Requests into the reordered Reletting Restrictions section, which has been moved from section 4.1 to section 5. |
| 1.3 | - Throughout | March 2026 | Sections 3.1, 3.3 and 3.4 have been consolidated into Section 3, which has been renamed from Occupational health and safety to Safety and inspection requirements. Content previously in Section 5.2 Asset Management oversight and approval has been renamed Vacancy durations and relocated to Section 10 Referral guide and assessment: Asset Management Branch (now Section 10.4). The table Reletting restrictions requiring Asset Management oversight and approval, originally part of Section 5.2, has been removed and incorporated into the Referral guide and assessment: Asset Management Branch content. A new Section 10.3 Reletting restriction and vacancy management has been created from the remaining relevant Section 5.2 content, retaining information on reletting restrictions requiring Asset Management Branch review before implementation, and clarifying local office responsibilities during the vacancy period. Section 5.3 Raising a reletting restriction for review and approval has been renamed Pre‑referral requirements and relocated under Section 10 Referral guide and assessment: Asset Management Branch (now Section 10.1). Section 5.4 CSHS Manager oversight and approval of referrals to Asset Management has been renamed CSHS Manager oversight and approval and relocated under Section 10 Referral guide and assessment: Asset Management Branch (now Section 10.2). Section 6 Goods left behind has been removed, as it solely redirected staff to the Goods Left Behind Operational Guidelines for instruction. Removing it streamlines the document and avoids unnecessary cross‑referencing. Section 9 Vacant unit referrals to Asset Management Branch has been introduced. Section 10 Referral guide and assessment: Asset Management Branch has been introduced. Tables have been introduced throughout the document to improve clarity and usability. Section 9 Move‑in Tenancy Condition Reports has been relocated to become a subsection under Vacant unit handback inspection (now Section 7.1). As a result of the amendments to Section 5 and subsequent sections, the document’s section numbering has been updated, as detailed above. |

# Contents

1	Revision history	4

Contents	7

2	Overview	8

3	Safety and inspection requirements	8

3.1	Properties presenting a risk of contamination	9

4	Target timeframes for vacant unit maintenance	10

4.1	Contractor extension requests	10

4.2	Impacts of delays and liquidated damages	10

4.3	Considerations for approving extension requests	10

5	Reletting restrictions	11

5.1	General manager oversight	11

5.2	HiiP tenancy task list	12

6	Vacant unit work	12

6.1	Mandatory lock changes	12

6.2	RTA safety checks	12

6.3	Standard work	13

6.4	Additional work	14

7	Vacant unit handback inspections	14

7.1	Tenancy Condition Reports	15

8	Guide to vacant unit checks	16

Minimum rental standards	16

Other vacant unit checks	19

9	Vacant unit referrals to Asset Management Branch	21

10	Referral guide and assessment - Asset Management Branch	23

Asset intent	23

Condition based referral check	24

Asset intent descriptions	27

10.1	Pre referral requirements	29

10.2	CSHS Manager oversight and approval	30

10.3	Reletting restrictions and vacancy management	31

10.4	Vacancy durations	31

# Overview

Vacant unit maintenance refers to the routine tasks and checks performed to ensure a rental property is clean, safe, functional, and ready for a new renter after the previous renter has moved out. This process helps maintain the property’s condition, address any wear and tear, and comply with legal standards before re-letting. The key objectives are to:

ensure the property remains in a suitable and compliant condition while unoccupied.

prepare the unit to meet the functional and safety needs of the next renter.

ensure the unit adheres to all safety, health, and minimum rental standards.

# Safety and inspection requirements

When conducting vacant unit inspections, staff must take appropriate precautions to protect their safety and minimise exposure to potential hazards. The following table outlines safety requirements that apply when inspecting vacant units:

Table 1: Safety requirements for vacant unit inspections

| Requirement | Description |
| --- | --- |
| Condition of entry due to unsafe conditions | Do not enter or proceed with the vacant unit inspection if: the property poses any immediate threat to health or safety illegal occupants are identified. unsafe conditions are observed such as: unstable flooring severe structural damage, or the presence of hazardous materials like asbestos. cluttered or obstructed pathways make it unsafe to move around. there are signs of contamination: methamphetamine pipes other drug paraphernalia, or evidence of pests etc. Refer to Section 3.1 Properties presenting a risk of contamination for more information. |
| Pre‑entry risk check | Assess the exterior and surroundings for hazards before entering (e.g. animals, damage, unsecured items, signs of illegal activity). |
| Announce entry | Knock and clearly announce entry to alert anyone inside and assess immediate risks before fully entering (e.g. animals, persons present, hazards). |
| Time spent inside | Remain in the property only as long as necessary to complete the inspection to minimise exposure to potential hazards. |
| Personal protective equipment | Wear appropriate Personal Protective Equipment, including: sturdy footwear shoe covers gloves masks, and eye protection, where necessary. |
| Avoid placing personal items | Do not place personal items, such as folders, handbags, car keys, on furniture, fixtures, or surfaces to prevent contamination from dust, allergens, or other potential hazards in the property. |
| Refrain from touching surfaces without gloves | Do not touch surfaces or objects without gloves to minimise exposure to contaminants that may be present. |
| Vacant unit checks guide | Refer to Section 9 Guide to Vacant Unit Checks to assist in supporting compliance with minimal rental standards during inspections. |
| Printed room identifiers | Use printed room identifiers when taking photos to clearly correlate each image with the corresponding part of the property. |
| Document findings thoroughly | Record all observations and findings in detail to maintain a comprehensive inspection report. |

## Properties presenting a risk of contamination

If a vacant property is assessed as potentially contaminated, based on prior information or observations during the vacant unit inspection, the external locks must be changed by the head contractor. The key must then be securely stored (e.g. in a lockbox at the property or the local office).

No further vacant works may proceed until the local office has completed all required work, including any necessary testing, specialist assessment, or remediation.

Table 2: Common contamination risks and required actions

| Type | Detail |
| --- | --- |
| Asbestos | If asbestos‑containing materials are suspected or identified, staff must follow all safety protocols and refer the property to specialist contractors for assessment and remediation. |
| Pest infestations | When signs of pests (e.g. rodents or insects) are identified, staff must arrange professional pest control services. |
| Mould | Where mould is detected, its extent should be assessed, and specialist remediation engaged to remove it safely. |
| Needles or drug paraphernalia | Staff must not touch or handle syringes or drug‑related items. These must be reported immediately and removed by biohazard services. |
| Rubbish and debris | Excessive rubbish or waste must be cleared by professional cleaning or waste removal services. |

For more information on managing methamphetamine contamination in vacant public housing properties, refer to the interim guidelines available on the Public Housing Resources SharePoint, Vacants page (internal link) https://dhhsvicgovau.sharepoint.com/sites/Publichousingresources/SitePages/vacants.aspx?web=1.

# Target timeframes for vacant unit maintenance

Vacant unit maintenance is a key performance indicator (KPI), tracking the efficiency of the process. Offers to new renters are typically made during this period, supporting readiness for occupation once maintenance is completed.

The target timeframe for raising vacant unit maintenance is within two working days of either the end of a Residential Rental Agreement, in accordance with the Residential Tenancies Act 1997 (RTA), or a Victorian Civil and Administrative Tribunal decision determining the property has been abandoned. Once the vacant works are scoped and raised in HiiP Repairs, the head contractor has 14 calendar days to complete the works.

## Contractor extension requests

Timely completion of vacant unit works helps reduce vacancy periods and make homes available for renters sooner. Contractors may request extensions on returning a vacant unit, and staff should assess these carefully to minimise operational impacts and maintain accountability, including applying liquidated damages where appropriate.

## Impacts of delays and liquidated damages

Delays in maintenance and turnaround times for vacant units create operational inefficiencies and reduce available housing stock. Key impacts include:

extended vacancy periods, increasing costs and slowing allocations

pressure on service delivery and renters waiting for housing

ongoing contractor performance issues if delays are not addressed

### Liquidated damages

Liquidated damages help hold contractors accountable for delays and recover costs arising from missed deadlines. They may be applied when:

the contractor fails to meet contractual completion dates without valid justification

delays cause financial loss to the department

application of damages aligns with the terms and conditions of the contract.

## Considerations for approving extension requests

When assessing a contractor’s extension request, staff must apply the requirements outlined in the following table:

Table 3: Requirements for assessing extension requests

| Requirements | Description |
| --- | --- |
| Verify supporting evidence | Confirm the contractor has provided enough information to explain the delay (e.g. photographs, reports, specialist assessments). |
| Consult relevant internal staff | Seek input from staff such as Field Services Officers to help validate the request and understand site conditions. |
| Record the decision | Document the reasons for approving or declining the request in HiiP Repairs to support transparency, consistency, and auditability. |
| Manager approval | All extension requests must be approved by Team Managers or equivalent delegated officers. |

Where delays are not justified, staff must also consider whether liquidated damages should be applied in line with contractual requirements and operational impacts.

# Reletting restrictions

A reletting restriction is applied to a vacant property in HiiP to prevent it from being offered to new renters. When applied, the property is flagged as unavailable for reletting (vacant untenantable). They are often applied when there are issues that need to be addressed before the property can be re-let, such as:

goods left behind

illegal occupants

extensive maintenance works required

strategic decisions regarding the property.

Applying reletting restrictions helps properties to be flagged appropriately. This process:

facilitates accurate reporting on asset utilisation

provides visibility into the reasons behind extended vacancies

helps prioritise property management actions to reduce vacancy periods.

While most reletting restrictions are managed locally, Homes Victorias Asset Management Branch, also apply reletting restrictions to support:

capital planning

property strategy

compliance works.

For more information on Asset Management Branch’s involvement in reletting restriction management, including oversight and approval, refer to Section 10.3 Reletting restrictions and vacancy management.

For a complete list of reletting restriction types and associated delegates, refer to the Reletting Restriction Matrix available on the Public Housing Resources SharePoint, Vacants page (internal link) https://dhhsvicgovau.sharepoint.com/sites/Publichousingresources/SitePages/vacants.aspx.

## General manager oversight

Team Managers, Client Support and Housing Services (CSHS) Managers, or their equivalents, are responsible for regularly overseeing the application and use of reletting restrictions to support:

accountability

efficiency

achievement of KPI targets.

The frequency of oversight should range from daily to weekly, depending on the volume of property vacancies at the local office. Oversight activities include:

tracking approval requests, expiry dates, and extension requests

reviewing reletting restrictions to confirm they remain appropriate and effectively utilised

removing reletting restrictions when properties are ready to be re-let.

## HiiP tenancy task list

HiiPs tenancy task list automates and manages housing processes by directing tasks through defined workflows. It streamlines operations like property information handling by integrating data and assigning tasks to staff. This function:

supports role-based access, and

facilitates timely notifications and approvals, with escalations appearing in the tenancy task list for staff action.

The task list supports reletting restriction management by automatically sending email notifications to the local office’s general mailbox when a reletting restriction is:

closed off in HiiP

(end-dating does not close a reletting restriction)

withdrawn (cancelled)

rejected by Asset Management

approaching expiry (five days prior).

Staff responsible for managing office emails must promptly forward them to authorised delegates to ensure timely action.

# Vacant unit work

## Mandatory lock changes

Changing the locks is a mandatory requirement for every vacant unit to ensure the property's security and safety. This practice is not only a best practice but also a regulatory requirement aimed at protecting renter safety. The following table outlines the SOR codes associated with the mandatory lock change work:

Table 4: Mandatory changing of locks SOR codes

| Location | SOR code | Description |
| --- | --- | --- |
| Front Door | GH055 | Re-pin 1 x cylinder & supply 2 keys alike (for all external doors) |
| Other Doors | GH056 | Re-pin cylinders to other doors |

## RTA safety checks

RTA Safety checks are not a legislated requirement for vacant unit works. However, as part of the vacant unit inspection, staff must check for:

The operational functionality for all:

appliances, safety devices, and energy supply systems, such as:

smoke alarms

cooking appliances

exhaust fans

lights/general power outlets

taps or hot water service (eg for the delivery of hot water).

If any damage or tampering is identified during the inspection, staff must arrange responsive maintenance to address the specific issue. Responsive maintenance in this context refers to repairs or replacements required to ensure appliances, safety devices, or energy systems are functioning correctly. It does not involve raising full safety checks as part of the vacant unit works, even when damage is identified.

Inspection and Test Reports (ITRs) for Gas and Electrical Safety Checks are obtained as part of scheduled or legislated safety check programs, separate from vacant unit works. Noting, while safety checks are not required during vacant unit works, staff must retain ITRs to comply with the Residential Tenancy Regulations 2021, as new renters may request a copy of these reports.

## Standard work

Standard vacant unit works generally include tasks such as:

cleaning

adjusting and tightening all screens on doors, cupboards, hinges, and fittings and

cleaning spouting and downpipes.

However, before staff raise these works, they should assess whether these types of standard works are necessary. For instance, while cleaning is generally regarded as standard vacant unit work, a visual inspection may reveal that not every room requires cleaning.

Common SOR cleaning codes staff may consider raising as part of their standard vacant unit works include:

Table 5: Common SOR codes and descriptions

| Location | SOR code | Description |
| --- | --- | --- |
| Multiple locations | CR070 | Clean wipe & vacuum all surfaces |
| Multiple locations | CR155 | Clean carpet by vacuuming- Rate per unit |
| Multiple locations | GA205 | Check adjust and tighten all screws to all doors, cupboard doors, handles and fittings etc |
| Multiple locations | CR202 | Remove contact from walls and cupboards only if required |
| Multiple locations | GA205 | Check and tighten all screws to all cupboards |
| Multiple locations | CR158 | Steam clean carpets - per room (more than 1 room) |
| Multiple locations | CR157 | Steam clean carpet - 1 room only |
| Kitchen | CR175 | Clean stove |
| Kitchen | CR188 | Clean kitchen complete - does not include floor |
| Kitchen | CR250 | Clean and scrub floors |
| Kitchen | CR195 | Clean exhaust fan only - not to be used w/cleaning |
| Laundry | CR210 | Clean linen laundry landing cupboards and built in robe |
| Laundry | CR211 | Clean laundry fixtures complete -including exhaust fan |
| Laundry | CR212 | Wash laundry complete- ceiling/walls etc |
| Bathroom | CR220 | Clean bathroom complete- include floor |
| Toilet | CR230 | Wash and scrub toilet complete including exhaust fan and floor |
| Hall | CR262 | Wash hall |
| Lounge | CR252 | Wash lounge and family room |
| Bedrooms | CR256 | Wash bedroom |
| External | RS270 | Clean out spouting |
| External | RS260 | Clean down pipe |

## Additional work

When considering additional maintenance works during a vacant property inspection, staff should apply a value‑for‑money approach. Table 14 outlines the key principles to guide decision‑making.

Table 6: Principles for assessing additional maintenance needs

| Principle | Description |
| --- | --- |
| Repair over replacement | Prioritise repairing items whenever feasible, rather than opting for full replacement. |
| Minimal aesthetic work | Limit work to the minimum necessary for aesthetic improvement, focusing on essential repairs that enhance the property’s appearance without unnecessary cost. |
| Targeted floor covering replacement | If carpet or vinyl in a room is damaged or marked beyond acceptable levels, replace only the affected area rather than the entire room. |
| Selective wall repair | Patch and repaint only the specific wall that is damaged or unable to be cleaned; avoid repainting the full room unless necessary. |
| Standardised door handle installation | If internal door handles do not match the departmental standard and one is missing, install a standard-compliant handle on the specific door only. |
| Odour management in carpets | If strong odours persist after steam cleaning, identify which rooms or sections of carpet are affected and replace only those areas. |

# Vacant unit handback inspections

As the property nears completion, the contractor will schedule a date and time for a vacant unit inspection. Staff are responsible for conducting a thorough inspection of the vacant unit alongside the contractor, reviewing all raised repair items. This includes cross-checking that all works have been completed to a satisfactory standard before accepting the unit as ready for reletting.

Expectations for staff during hand back inspections include:

Thorough review: Inspect all areas of the unit to ensure repairs align with what was agreed upon in the scope of works. Pay attention to common issues like:

cleaning,

safety features

compliance with minimum rental standards

quality of work.

This includes checking for functionality (e.g., working plumbing, locks, and appliances).

Staff must not accept a hand-back of the property if works remain incomplete or do not meet required standards. Clearly document any outstanding or unsatisfactory work and communicate this to the contractor for rectification.

## Tenancy Condition Reports

Move-in Tenancy Condition Reports are a mandatory requirement in the sign-up process, as stipulated by the RTA. These reports must be completed by staff during the vacant unit handback or prior to reletting a property. They support accurate documentation of the property's condition and help establish its state at the time of occupancy.

For more information, refer to the Tenancy Condition Reports Operational Guidelines https://providers.dffh.vic.gov.au/tenancy-management-operational-guidelines.

# Guide to vacant unit checks

To uphold the integrity of rental properties and support compliance with minimum rental standards, this guide aims to provide actionable guidance for staff undertaking inspections, in alignment with Section 68 of the RTA. During vacant unit inspections, staff should conduct visual checks to confirm that all requirements are met. If staff are unable to visually verify compliance, they should seek further guidance from relevant local office staff, such as Field Services Officers.

*Note: All rental properties must meet 14 categories of minimum rental standards, and rental providers are responsible for ensuring compliance with these standards on or before the day a renter moves in.

## Minimum rental standards

| Property address | Click or tap here to enter text. |
| --- | --- |

Table 7: Minimum rental standard requirements

| Category | Requirement | Met | Not met |
| --- | --- | --- | --- |
| Bathrooms | The bathroom must have a: washbasin shower or bath, and all must have a reasonable supply of hot and cold water. Showers must have a shower head with a 3-star water efficiency rating. If this is not possible due to the property’s age, a 1- or 2-star rating is acceptable. | ☐ | ☐ |
| Electrical safety | Rental properties must have: modern switchboards with circuit breakers and electrical safety switches, known as Residual Current Devices (RCDs), installed. Note: This requirement is already addressed by the RTA safety check, which has either been completed or is scheduled for completion soon. As part of the vacant unit work, staff only need to visually inspect appliances such as ovens, stoves, hot plates, heating and cooling systems, hot water services, exhaust fans, lights, and general power outlets to confirm they are operational and functional. | ☐ | ☐ |
| Heating | All rental properties are required to have a fixed heater (not portable) in good working order located in the main living area. For rental agreements entered into on or after 29 March 2023, this heater must be energy efficient. If an existing fixed heater is not energy-efficient, the rental provider is responsible for upgrading it. An energy efficient fixed heater must be one of the following: a non-ducted air conditioner or heat pump with a 2 star or above energy rating a gas space heater with a 2 star or above energy rating a ducted heating or hydronic heating system with an outlet in the main living area | ☐ | ☐ |
| Kitchen | The property must have a: kitchen with a dedicated cooking and food preparation area sink connected to hot and cold water, and stovetop with two or more burners. - If an oven is present, it must be in good working order. - These requirements do not apply if the property is listed in the heritage register at Heritage Council Victoria https://heritagecouncil.vic.gov.au/protecting-our-heritage/heritage-protection-process/victorian-heritage-register and has an approved exemption from the standard. | ☐ | ☐ |
| Laundry | If the property has a laundry, it must be connected to a reasonable supply of hot and cold water. | ☐ | ☐ |
| Lighting | Inside rooms, corridors and hallways must have access to light to make the areas functional. During the day, natural light can include light borrowed from an adjoining room. At night, renters should have access to artificial light. These requirements do not apply if the property is listed in the heritage register at Heritage Council Victoria https://heritagecouncil.vic.gov.au/protecting-our-heritage/heritage-protection-process/victorian-heritage-register and has an approved exemption from the standard. | ☐ | ☐ |
| Locks | The property’s external entry doors must have functioning deadlocks (a ‘deadlock’ is defined as a deadlatch with at least one cylinder) or be fitted with locks that can be unlocked with a key from the outside but can be unlocked without one from the inside. The only cases where a deadlock doesn’t have to be fitted to a door are when: a door cannot be secured with a deadlock - for example, because of its position it is a screen door in the same door frame as an external door a different type of lock or device is required under another Act or law the door is not accessible because there is another type of security barrier. For example, a locked door to an apartment building, or a locked gate the property is registered under the Heritage Act 2017 and has an approved exemption from the standard. Read more about locks and security on the Consumer Affairs Victoria website https://www.consumer.vic.gov.au/housing/renting/repairs-alterations-safety-and-pets/keeping-the-property-safe/locks-and-security. | ☐ | ☐ |
| Mould and damp | All rooms must be free from mould and damp caused by or related to the building structure. | ☐ | ☐ |
| Structural soundness | The property must be structurally sound and weatherproof. | ☐ | ☐ |
| Toilets | The toilet must be in good working order and connected to a sewage system, wastewater treatment system, or approved local council system. | ☐ | ☐ |
| Ventilation | Rental properties must have adequate ventilation in all habitable rooms, including the bathroom, shower, toilet, and laundry, meeting the requirements of the Building Code of Australia https://www.abcb.gov.au/resources | ☐ | ☐ |
| Vermin-proof bins | Rental providers must supply a rubbish and recycling bin that is vermin-proof and meets council collection standards. | ☐ | ☐ |
| Window coverings | Windows in rooms likely to be used as bedrooms or living areas must be fitted with curtains or blinds that can be closed, block light and provide privacy and made safer with a curtain and blind safety kit. | ☐ | ☐ |
| Windows | All external windows capable of opening must have a functioning latch to secure the windows and allow them to be left open or closed. | ☐ | ☐ |

## Other vacant unit checks

Table 8: Other vacant unit checks to ensure the property is well maintained and complies with minimum rental standards

| Category | Item | Description | Met | Not met |
| --- | --- | --- | --- | --- |
| External doors | Security screen doors | All external doors must have a security screen door. | ☐ | ☐ |
| General | Lights | All lights must be functioning properly. | ☐ | ☐ |
| General | Exposed wiring | There must not be any exposed wiring. | ☐ | ☐ |
| General | Appliances | All appliances must power on and function correctly without issues. Check for unusual noises or odours and inspect for visible damage to appliance exteriors. | ☐ | ☐ |
| General | Hot water system | There must not be any leaks from the water heater. Check for pooling or dripping. Confirm hot water is available at all taps and that it maintains consistent temperature. | ☐ | ☐ |
| General | Smoke alarms | Smoke alarms must be fitted and operational. | ☐ | ☐ |
| General | Carbon monoxide alarms | Verify carbon monoxide alarms are fitted (if applicable) and operational. | ☐ | ☐ |
| General | Telephone line and NBN connection | Verify that a telephone line is connected (if applicable). Ensure National Broadband Network (NBN) is connected where required. | ☐ | ☐ |
| General | Signs of mould/dampness | Check for visible signs of mould or damp caused by structural issues. Look for water penetration signs in ceilings, walls, floors, coverings, and cupboards. | ☐ | ☐ |
| General | Non-standard modifications safety | Check for any non-standard modifications or fixtures for safety/legal compliance. | ☐ | ☐ |
| General | Asbestos inspection | Inspect for obvious cracked/damaged asbestos materials. Ensure safety protocols are followed if found. | ☐ | ☐ |
| General | Structural integrity | Inspect walls/floors for noticeable slope or cracks/gaps greater than 10mm wide. Ensure overall structural integrity is sound. | ☐ | ☐ |
| General | Pest infestation | Inspect for visible signs of pest infestation or biological hazards. Take necessary pest control measures if needed. Look for termite damage or significant rot/decay in building timbers. | ☐ | ☐ |
| Floors/walkways | Floor coverings safety | All floor coverings must be free from trip hazards. | ☐ | ☐ |
| Kitchen/bathroom/utility area | Taps functionality | Check all taps operate smoothly without dripping. Check for consistent water pressure from all fixtures. | ☐ | ☐ |
| Kitchen | Kitchen cupboards/drawers functionality | Check that all kitchen cupboards and drawers open/close easily and have handles. | ☐ | ☐ |
| Rooms | Internal doors with hardware | Check all rooms have doors equipped with door hardware, but internal doors should not have locks or latches. | ☐ | ☐ |
| Rooms | Flywire screens | Check at least one window in each room has flywire fitted and in good repair. | ☐ | ☐ |
| Bathroom | Bathroom cupboards/drawers functionality | Check bathroom cupboards and drawers open/close without difficulty and have handles. | ☐ | ☐ |
| Yard/exterior | Functional letterbox and clothesline/shed | Confirm property has a functional letterbox, clothesline, and garden shed/storage locker if applicable. | ☐ | ☐ |
|  | Tree proximity to external walls | Ensure no trees are within 1 meter of external brick walls; check for trees/shrubs impeding access or impacting structures. | ☐ | ☐ |
|  | Swimming pools | Confirm no swimming pools or external spas are present. | ☐ | ☐ |

# Vacant unit referrals to Asset Management Branch

When assessing a vacant property for escalation, staff must identify which works fall within local office responsibility and capacity, and which require involvement from Homes Victoria’s Asset Management Branch.

The following tables outline the different types of works, clarify responsibility for each, and provide examples to assist in determining whether a referral is required. Staff must also refer to Section 10 Referral guide and assessment – Asset Management Branch when determining responsibility and prior to submitting a referral.

Table 9: Capital works (Asset Management Branch responsibility)

| Category | Description |
| --- | --- |
| Definition | - Significant repairs, upgrades or replacements that restore or improve the long-term function of the property. These works usually exceed the financial and operational capacity of local offices. |
| Delivery pathways | - Programmed capital works delivered annually by Homes Victoria. - Minor capital works delivered locally under the Schedule of Rates (SOR) or as Not on Schedule (NOS) items. |
| Examples of capital works | - Structural repairs: major roof replacement, structural wall repairs, extensive flooring replacement. - Compliance works: fire safety upgrades, asbestos removal, accessibility modifications. - Extensive damage repairs: fire‑damaged properties, major vandalism repairs, methamphetamine/mould/pest contamination remediation. - Terrain and site works: retaining walls, major drainage works, significant landscaping or earthworks. |
| Referral requirement | Vacant properties requiring this level of work must be referred to the Asset Management Branch for assessment. |

Table 10: Non-capital works (Local office responsibility)

| Category | Description |
| --- | --- |
| Definition | Day-to-day maintenance and minor repairs needed to keep a property safe, functional, and ready for reletting. These works do not extend the asset’s life and are carried out locally. |
| Delivery pathway | Local housing offices raise work orders under the SOR for vacant maintenance and fair wear-and-tear repairs. |
| Examples of non‑capital works | - General maintenance: lock changes, repairing windows, fixing taps. - Cleaning and waste removal: rubbish removal, pest control, internal and external cleaning. - Minor repairs: patching walls, securing loose fixtures, partial gutter replacements, minor flooring repairs. - Safety and security: fence repairs, reattaching cupboard doors, replacing smoke alarm batteries, fixing broken tiles or benchtops. |
| Referral requirement | These works should be completed locally. They do not require referral to the Asset Management Branch. |

In some situations, routine maintenance may uncover underlying issues that require Capital Works. For example, a leaking roof may be repaired locally, but a Field Services Officer inspection may reveal that the roof is significantly deteriorated and requires full replacement. When this occurs:

- local offices should complete the immediate non‑capital maintenance needed to address the presenting issue (e.g. repairing the active leak).

- if further investigation identifies major or extensive works, the local office must document the findings and submit a referral to the Asset Management Branch for assessment.

# Referral guide and assessment - Asset Management Branch

| Property address | Click or tap here to enter text. |
| --- | --- |

| Property PIN | Land PIN | Year built | Cladding type | Demand and supply |
| --- | --- | --- | --- | --- |
| Click or tap here to enter text. | Click or tap here to enter text. | Click or tap here to enter text. | Click or tap here to enter text. | Choose an item. |

## Asset intent

Select the relevant asset intent. Refer Asset intent descriptions for more information.

| Asset intent | Referral requirements |
| --- | --- |
| Choose an item. | Where the asset intent is ‘Hold and Improve’ or ‘Hold and Maintain’, the referral must be submitted via the OurService Upgrade Referral Request eform (internal link) https://ourservice.support.vic.gov.au/ourservice?id=sc_cat_item&sys_id=5bfb744adb6a2110ddf5bc56f39619b1&sysparm_category=dfeb09fe1bc52110af5d98ef0d4bcb79. Where the asset intent is ‘Review’, ‘Sell’ or ‘Redevelop’, the referral must be submitted via the OurService Disposal Referral Request eform (internal link) https://ourservice.support.vic.gov.au/ourservice?id=sc_cat_item&sys_id=edc96f261b2e6510af5d98ef0d4bcbf9. |

## Condition based referral check

Is the property affected by any of the following conditions? (Select all that apply). If any listed condition applies, use the OurService Upgrade Referral Request eform (internal link) https://ourservice.support.vic.gov.au/ourservice?id=sc_cat_item&sys_id=5bfb744adb6a2110ddf5bc56f39619b1&sysparm_category=dfeb09fe1bc52110af5d98ef0d4bcb79.

| Condition category | Yes / No | Referral required | Criteria / referral requirements | Supporting documents attached | Referral notes |
| --- | --- | --- | --- | --- | --- |
| Fire damage | Yes ☐ No ☐ N/A ☐ | Yes | All fire‑damaged properties must be referred via OurService. | Yes ☐ No ☐ N/A ☐ | Click or tap here to enter text. |
| Renter damage | Yes ☐ No ☐ N/A ☐ | Case‑by‑case | Renter damage may be identified where deterioration or damage is caused by the actions, misuse, or neglect of renters or their visitors. Referral applies when: the damage impacts structural integrity, safety, or RTA compliance specialist trades or significant rectification are needed. | Yes ☐ No ☐ N/A ☐ | Click or tap here to enter text. |
| Vandalism / squatters | Yes ☐ No ☐ N/A ☐ | Case‑by‑case | Vandalism or squatter damage may be identified where unauthorised individuals have caused damage to the property, affecting security or habitability. Referral applies when: the damage is significant or affects essential services, safety, or structural elements specialist rectification or security upgrades are required. | Yes ☐ No ☐ N/A ☐ | Click or tap here to enter text. |
| Methamphetamine contamination | Yes ☐ No ☐ N/A ☐ | Case‑by‑case | Methamphetamine contamination is identified when local testing confirms the presence of contamination requiring remediation. Referral applies when: remediation involves significant strip‑out works contamination impacts habitability or compliance. | Yes ☐ No ☐ N/A ☐ | Click or tap here to enter text. |
| Early degradation | Yes ☐ No ☐ N/A ☐ | Case‑by‑case | Early degradation may be identified where the level of deterioration is beyond what the local office can reasonably repair. Referral applies when: specialist trades or structural rectification are needed the issue is likely to escalate without broader asset intervention. In addition, the deterioration must not be: due to renter damage caused by fire, contamination, or vandalism the result of an isolated incident, but instead systemic or building‑related. | Yes ☐ No ☐ N/A ☐ | Click or tap here to enter text. |
| Structural repairs | Yes ☐ No ☐ N/A ☐ | Case‑by‑case | Structural repair needs may be identified when damage or deterioration affects the building’s structural integrity or safety. Referral applies when: works relate to foundations, load‑bearing elements, framing, or other structural components engineering assessment or specialist trades are required the extent of structural work exceeds the high‑cost vacant threshold. Structural issues must not be managed locally where safety or structural integrity is compromised. | Yes ☐ No ☐ N/A ☐ | Click or tap here to enter text. |
| Full upgrade is required due to major issues not related to fire, renter damage or contamination | Yes ☐ No ☐ N/A ☐ | Case‑by‑case | A full upgrade may be identified when the property has significant deterioration or functional failure requiring comprehensive renewal. Referral applies when: multiple building elements have reached end‑of‑life or require replacement the scale of works exceeds local office capacity specialist or coordinated upgrade planning is required. | Yes ☐ No ☐ N/A ☐ | Click or tap here to enter text. |
| Age related upgrade | Yes ☐ No ☐ N/A ☐ | No (unless unsafe) | Age‑related upgrade need may be identified where the property is safe and habitable but approaching the point where major renewal will soon be required. Referral is not required unless safety is compromised. Where the property remains compliant with the RTA, staff should: relet the property submit a request for inclusion in future upgrade programs via OurService. | Yes ☐ No ☐ N/A ☐ | Click or tap here to enter text. |

## Asset intent descriptions

| Asset intent | Description |
| --- | --- |
| Hold and improve | This intent applies where the property meets Homes Victoria’s asset objectives and continues to be a productive asset to be maintained and potentially improved. |
| Hold and maintain | This intent applies where the property is older and close to the end of its asset lifecycle but requires minimal maintenance or periodical renewal. At the end of its lifecycle, the site may be suitable for redevelopment or sale. The property should be maintained to meet the requirements of the Residential Tenancies Act, including Minium Rental Standards. |
| Redevelop | This intent applies where the land should be retained as it meets Homes Victoria’s overall objectives. Redevelopment may: retain and redevelop the site for social housing, including planning, demolition and construction of new dwellings; or retain the land for commercial objectives, such as commercial retail or ground lease options. |
| Sell | This intent applies where the property does not adequately meet Homes Victoria’s asset objectives or is not financially prudent to invest in (e.g. lower‑demand locations, limited or expensive redevelopment options, neighbourhood fatigue, site limitations, or restrictive/prohibitive planning controls) |
| Review | This intent applies where a more detailed review is required to determine the appropriate intent for the property. Examples include: the property is at end of life but still occupied development opportunities exist on contiguous sites that are occupied alignment is required with precinct development or neighbourhood strategy the property is part of broader work to confirm intent for a precinct or walk‑up block and depends on a realisation plan significant work is needed to determine intent for a complex precinct. |

Properties should not be referred to the Asset Management Branch when:

the issue can be addressed through standard make‑safe or minor repairs

costs fall within local office repair capacity

the CSHS manager has not reviewed and approved the referral

relevant information required has not been provided (e.g., year built, cladding type)

pre-referral vacant unit work is not complete

the required documentation for the condition category has not been attached to the referral.

For subsequent procedures refer to:

Section 10.1 Pre referral requirements

Section 10.2 CSHS Manager oversight and approval

Section 10.3 Reletting restrictions and vacancy management

## Pre referral requirements

When properties require substantial interventions beyond the scope of routine maintenance or local office capacity, staff and managers should determine if the Asset Management Branch’s involvement is required to support appropriate escalation.

Before raising and escalating a reletting restriction and an OurService referral, including product type changes, staff must first initiate the necessary vacant unit works, which include:

Table 11: Required actions before raising a reletting restriction and referral

| Requirement | Description |
| --- | --- |
| Secure the property | Change all locks and place keys in the on-site key safe. |
| Remove all rubbish and goods | Remove all rubbish and personal goods from internal and external areas, including sheds |
| Arrange a basic clean | Raise work orders for a clean based on the property’s condition and complete the clean as required. |
| Address suspected meth contamination | If methamphetamine contamination is suspected, obtain an Occupational Hygienist report and remediation quotation. |
| Prepare cost estimates | Provide an estimate for vacant unit works, referencing the Schedule of Rates where applicable. |

Staff must also compile comprehensive information for a thorough and well-documented referral, including:

Table 12: Required documentation for escalation

| Documentation | Description |
| --- | --- |
| Explanation of reletting restriction | Detailed notes outlining the reasons for the restriction, including assessments and decisions made. |
| Field Service Officer report | Assessment of the property’s condition. |
| Council Emergency Orders | Include where relevant, particularly for fire-damaged properties. |
| Methamphetamine reports and costs (if applicable) | Certified Occupational Hygienist report, including the testing report and remediation action plan Field Services Officer property report, including the Property Survey Report (PCA001) Methamphetamine remediation cost estimate worksheet. Remediation cost estimates must not include the cost of vacated maintenance works or the removal of rubbish that can be disposed of through normal municipal waste services. |
| Photographs | Internal and external images clearly showing the issues prompting escalation. |
| Other relevant documentation | Any additional reports or evidence relating to the property’s condition. |
| Vacancy checklist | Completed checklist confirming the property’s status and condition. |
| Referral guide and assessment - Asset Management Branch | Completed referral guide to the Asset Management Branch, where applicable. |

Once the above has been completed, staff must simultaneously:

Table 13: Actions to complete after prerequisites are finalised

| Action | Description |
| --- | --- |
| Apply and escalate the reletting restriction in HiiP | Apply and escalate the relevant reletting restriction within 24-48 hours of identifying the need for Asset Management involvement. Include the OurService reference number in the comments to support accurate tracking and follow-up. |
| Initiate an OurService referral | Submit a referral through the OurService Catalogue under Housing Services, selecting the request type appropriate to the situation. |
| Submit for CSHS Manager review and approval | Forward the reletting restriction and referral through the HiiP and OurService workflows to the CSHS Manager (or equivalent) for review and approval. |

## CSHS Manager oversight and approval

Before approving a referral, CSHS Managers must first determine if works can be completed locally. This includes confirming that:

Table 14: CSHS Manager requirements before approving a referral

| Requirement | Description |
| --- | --- |
| Assess local capability | Confirm whether the work can be completed locally, including: whether repairs are minor, and within local cost limits |
| Review supporting evidence | Review: Field Service Officer reports cost estimates, and risk assessments to confirm they clearly show whether the work can be managed locally or requires escalation. |
| Confirm reletting restriction status | Validate a RAM-endorsed reletting restriction has been applied in HiiP, with accurate and complete notes outlining the reason for escalation. |
| Validate referral pathway | Validate the referral aligns with the correct path based on: asset intent property type, and cladding considerations. |
| Confirm OurService referral completeness | Make sure all relevant information has been entered into the OurService form, with an accurate description and the correct request type selected. |
| Verify required documentation is attached | Confirm submission includes: Field Service Officer report(s) council Emergency Order (if applicable) methamphetamine remediation reports and costings (if required) internal and external photographs showing the issues any other relevant reports or documentation completed vacancy checklist |
| Record recommendation and decision | Provide recommended upgrade reason(s), include a brief description of the property’s condition, and approve or decline the referral in a timely manner. |

If these conditions are met, the referral should not be approved and the work managed locally.

## Reletting restrictions and vacancy management

If a referral is accepted by the Asset Management Branch, the associated reletting restriction may be applied. The local office retains responsibility for vacancy management tasks, including:

securing the property in the event of any incidents

maintaining the grounds, including grass cutting, and

taking appropriate action against illegal occupants, if necessary.

## Vacancy durations

When a property is referred for assessment or works, it may remain vacant for an extended period due to the nature of the works or decisions involved. The typical vacancy durations are:

Table 16: Typical vacancy durations

| Duration | Description |
| --- | --- |
| 30 days | Usually applies to: new constructions that have reached approximately 75% completion recently acquired properties, and properties under review by Asset Management |
| 60 days | For relocation projects managed centrally. |
| 90 days | When handover of new construction properties is delayed due to postponed service connections. |
| 185 days | For properties: requiring major works earmarked for future redevelopment, or inclusion in the sale pipeline. |
| 365 days | For properties undergoing fire rectification works. |
| No fixed end date | For properties: approved for redevelopment or sale, or undergoing major upgrades such as full restoration after fire damage |